Kermit in Pygmy

A Simple Implementation of the Kermit Protocol in Pygmy Forth

by Frank Sergeant

Introduction

Kermit is not only the name of Henson Associates Inc's

famous frog but also the name of an extensive and complex

file transfer/remote computer access application written by

Columbia University and/or volunteers (as I understand it)

and sold and distributed by Columbia University [1].  The

name Kermit is also used to refer to the family of file

transfer protocols used in Columbia University's Kermit and

by many modem programs.  As I use the term Kermit throughout

the rest of this article, it refers to the Kermit protocols,

particularly the simplest form implemented here in Pygmy

Forth.

My Forth applications sometime need to transfer files

with other systems via modem.  I felt I needed to implement

one or more of the following protocols: XMODEM, YMODEM,

Kermit, ZMODEM.  The basic XMODEM protocol doesn't have much

respect these days, as it does not transmit exact file

lengths, but rounds up to the nearest block size (128

bytes).  Enhanced versions of XMODEM, such as YMODEM

overcome this.  Kermit is well thought of and widely

available.  ZMODEM is usually considered to be the best

although there seem to be arguments between the kermit and

ZMODEM camps as to which is the better, faster protocol

under various circumstances.  I, personally, found the

ZMODEM arguments more persuasive, but I don't really care

about minor differences.  I wanted something that worked

reasonably well and was fairly easy to implement.  Looking

over the Kermit and ZMODEM protocols, I decided that a

simple form of Kermit would be slightly easier to program.

Later, I hope to implement ZMODEM as well.

Why Write My Own

You may well ask why we insist on writing our own

version instead of using an existing product.  Indeed, we

have experimented some with other products and have been

severely disappointed.  For example, we have had a lot of

trouble with PCAnywhere trying to get it configured

correctly under DOS.  On some client systems, we were unable

to configure it by changing the configuration file in the

directory PCAnywhere would be run from.  Instead, we had to

resort to a Rube Goldberg (an extremely cumbersome) approach

to get the configuration to "take."  We had to modify the

batch file temporarily so it would not call the PCAnywhere

script, run the application and let it shell out to

PCAnywere, change and save the settings, and again modify

the batch file so it would run the script.  We never found

just why we could configure it in some offices without going

through this tedious process but not in most, but it was

enough to make us hate PCAnywhere.

Further, while the modem scripts would run under DOS,

PCAnywhere was too slow.  Also, we could not make the DOS

versions of the scripts run under the Windows 95 version of

PCAnywhere.  Also, it was not free and we needed to fool with

getting each client to purchase PCAnywhere.  Similarly, the

use of Columbia University's Kermit product would have

required each customer to purchase a license for it (yes, it

is NOT free).  Ditto for the Oman Technologies ZMODEM

product.  So, to hell with the third-party products.  We

will just write our own, have full control of it, simpler

installation, and a faster user interface.  Were this

running under Unix/Linux, we would have used the built-in,

freely distributable zmodem.

The Kermit Protocol

My main guide for the Kermit protocol was _C

Programmer's Guide to Serial Communications_ by Joe Campbell

[2].  Note, there are some errors in Campbell's examples.

Files are transferred in Kermit by exchanging "frames"

between the sender and the receiver.  Every frame begins

with an SOH (start of header) character and ends with a

carriage return.  Between them are the following five

fields: length, sequence, type, data, checksum.  Each

field except for the data field is a single character in

printable form.  The process of converting a number, say

for the length field, into a character, is called

"character-ization" and is done by adding the value of

the space character to the number.  Thus, the number zero

becomes $20 and prints as a space.  The number thirty-seven

(i.e. $25) becomes $45 and prints as the letter "E" and so

forth.  I use the words CHAR and UNCHAR to convert between

the two formats.  Note this requirement of fitting numbers

into a single byte as a printable character limits the

numbers to the range zero to 94 and limits the size of

frames that can be transmitted.  Various extensions to the

basic Kermit protocol allow longer frames, but not the

simple form described here.

The frame types used are S to initiate a file transfer

session, F to send a file name, D to send a data frame,

Z to indicate end of file, B to indicate end of

transmission, E to indicate a fatal error, A to send file

attributes (we do not use this one), Y to ack a frame, and

N to nak a frame.  No single bytes are exchanged between the

sender and receiver, only whole frames.  The length field

indicates the number of bytes to follow the length field up

to and including the checksum field, but not the carriage

return.

To transfer a file, the sender waits for an N-frame (a

negative acknowledgement), then sends an S-frame to tell the

receiver what values it wants to use for various protocol

parameters, such as maximum frame length, the repeat

character, the escape character, etc.  The receiver then

sends a Y-frame (an acknowledgement) with its preferred

values for these parameters.  The word COMPROMISE takes the

more conservative value for each of the parameters.  This

single exchange of frames sets the protocol parameters for

the session.  In the S-frame and its Y-frame, the values of

the parameters are sent in a fixed-order in the frames' data

fields.

Kermit has two major ways of converting bytes before

transmitting them.  One is the "character-ization" already

mentioned, for converting numbers.  The other is

"controlification" to flip a bit in a control character to

turn it into a printable character.  Numbers, as such, are

expected in the length and sequence and checksum fields, but

never in the data field (except during the initial S and Y

frames that establish the protocol parameters).  Kermit

would allow for transmitting 7-bit data bytes by escaping,

with an ampersand, each byte with a high bit set and then

clearing that high bit, but we do not do this.  We assume

the availability of an 8-bit channel.  Kermit does insist,

though, on escaping control characters (with "#") and at

least some implementations insist on compressing data by

run-length encoding repeated bytes.  "#" and "~" characters,

when appearing in the data as themselves, must be escaped.

Thus a single "#" would appear in the data field as "##".

To keep things simple, we never compress the data when

we are sending a file.  Unfortunately, not all

implementations of Kermit respect the request not to use

repeat counts.  Therefore, we must be prepared to handle

compressed data when receiving a file.  (Since our main

purpose in using Kermit is to transmit zip files, it doesn't

look like we would gain much speed by compressing the data

we send.)

Kermit allows various types of checksums, ranging from

a single byte to three bytes.  Our main use will be to

transmit zip'd files, with the zip file itself providing an

additional level of data integrity verification with its

32-bit CRC, therefore, I am content to use a 1-byte

checksum, the simplest form Kermit allows.

The V-frame

We invent a special input frame and assign it type V.

A V-frame is never actually sent.  Instead, whenever a

timeout occurs, KSER-IN terminates (_and_ terminates its

caller GETFRAME) and returns a V-frame.  This way, a timeout

is not a special case, but just an ordinary "frame."

What Else Can Go Wrong

In addition to a timeout, a frame with a bad checksum

might be received.  In this case, we send an N-frame to

alert the sender so it will try again.  If we are receiving

and a frame is lost, the V-frame alerts us to the missing

frame and we again send an N-frame to request a repeat.

When sending, if a frame is lost or its Y-frame is lost,

either a timeout or receipt of an N-frame alerts us to the

need to resend that frame.  When we receive a duplicate

frame (perhaps because our Y-frame never reached the

sender), we basically ignore it.  However, we do acknowledge

it so the sender will be free to continue.  The progress of

a file transfer is indicated on-screen by the printing of a

dot for each frame transferred.  Our code will wait forever

attempting a transfer unless it is cancelled by the user or

unless it receives an E-frame indicating a fatal error.

The Environment

This implementation expects the SER-IN, SER-OUT and

related words that allow transmitting and receiving to and

from the serial port connected to a modem.  "The PC Serial

Port in Forth" [3] describes one approach to handling the

serial port on a PC using interrupts.

My current approach does not use interrupts.  Instead,

it uses the new multi-tasking ability of the upcoming Pygmy

version 1.5.  The serial input is serviced by a separate

task, thus greatly simplifying the serial port code.

The Kermit code presented here should work regardless

of which form of serial handling is used.

How to Transfer Files

Assuming the modem connection has been made and the

other end is expecting to send or receive the file using

Kermit, type " filename.zip" SEND to send a file, or type

RECEIVE to receive a file.  The RECEIVE routine is capable

of receiving multiple files.  The SEND routine transmits a

single file.  Since Pygmy 1.5 accepts instructions from

the command line, you could type

PYGMY " filename.zip" SEND BYE

on the DOS command line or in a batch file, etc.  In our

application, we have a menu and a terminal mode with PgUp

and PgDn keys invoking the SEND and RECEIVE routines.  You

can see a clue as to how we use this in the definition of

KSER-IN on block 12007 where a user abort of the file

transfer executes the DEFER'd word MYMENU to put the user

back at the application menu.

Conclusion

This code has been used successfully at a number of

client sites, but, remember, it uses only a very basic

version of the Kermit protocol.  It very well could have

problems transferring between certain sites.  If you try it

out, please let me know your results.  For your downloading

convenience, I have zip'd the Kermit source and shadow

blocks, along with a copy of this article, and a preliminary

version of PYGMY.COM that contains the multi-tasking version

of the serial port words, and placed this on my web site as

http://www.eskimo.com/~pygmy/forth/kermit.zip.

References

[1] "Kermit: A File-transfer Protocol for Universities,"

parts 1 and 2, Frank da Cruz and Bill Catchings, June and

July, 1984, Byte magazine.

[2] _C Programmer's Guide to Serial Communications_, Joe

Campbell, Howard W. Sams & Company, 1987, ISBN

0-672-22584-0, pp 98-113.  Note, there are some errors in

Campbell's examples.

[3] "XT Corner: The PC Serial Port in Forth," Frank

Sergeant, _The Computer Journal_, issue 79, Fall 1996,

pp 5-10.  My web page, http://www.eskimo.com/~pygmy contains

a link to TCJ's web page.

Author's Bio

Frank Sergeant, on the "thirty year plan," received his

Master of Science in Computer Science degree from Southwest

Texas State University.  The degree and his 4.0 GPA were

hard won.  His thesis was entitled "Calculating the Crust of

a Set of Points in C++" and has nothing to do with Forth

except to help illustrate that computational geometry is

easier in Forth than in C++.  He can be reached at

pygmy@pobox.com.

END